程序代码：

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 2019/10/21 11:44:09

// Design Name:

// Module Name: experiment_3

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module experiment_3(

input [2:0] a,

input [2:0] b,

input c0,

output [2:0] sum,

output c3

);

wire [2:0]g;

wire [2:0]p;

wire [1:0]c;

assign g=a&b;

assign p=a^b;

assign c[0]=g[0]|(p[0]&c0),c[1]=g[1]|(p[1]&c[0])|(p[1]&p[0]&c0);

assign c3=g[2]|(p[2]&g[1])|(p[2]&p[1]&g[0])|(p[2]&p[1]&p[0]&c0);

assign sum[0]=p[0]^c0;

assign sum[1]=p[1]^c[0];

assign sum[2]=p[2]^c[1];

endmodule

仿真：

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 2019/10/21 11:55:33

// Design Name:

// Module Name: test3

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module test3(

output [2:0] sum,

output c3

);

reg[2:0] in1;

reg[2:0] in2;

reg c0;

initial

begin

in1=3'b000;

in2=3'b000;

c0=1'b0;

end

always #10

begin

in1=in1+1;

in2=in2+1;

c0=c0+1;

end

experiment_3 t(in1,in2,c0,sum,c3);

endmodule

约束：

set_property IOSTANDARD LVCMOS33 [get_ports {a[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {a[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {a[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {b[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {b[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {b[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sum[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sum[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sum[0]}]

set_property PACKAGE_PIN P5 [get_ports {a[2]}]

set_property PACKAGE_PIN P4 [get_ports {a[1]}]

set_property PACKAGE_PIN P3 [get_ports {a[0]}]

set_property PACKAGE_PIN P2 [get_ports {b[2]}]

set_property PACKAGE_PIN R2 [get_ports {b[1]}]

set_property PACKAGE_PIN M4 [get_ports {b[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports c0]

set_property PACKAGE_PIN N4 [get_ports c0]

set_property PACKAGE_PIN F6 [get_ports c3]

set_property PACKAGE_PIN G4 [get_ports {sum[2]}]

set_property PACKAGE_PIN G3 [get_ports {sum[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports c3]

set_property PACKAGE_PIN J4 [get_ports {sum[0]}]